Food Spoilage Detection: An IoT-Driven Machine Learning System for Predicting Remaining Shelf Life

A project report

submitted by

Shreyas Patel

(2023KUEC2015)

Shikhar Singh

(2023KUEC2021)

Rudrawar Parth

(2023KUEC2023)

under the supervision of

Dr. Ashok Kumar Kherodia

in partial fulfillment of

the requirements for the degree of

Bachelor of Technology

in Electronics and Communication Engineering

to the

Department of Electronics and Communication Engineering

Indian Institute of Information Technology Kota

May 2026

DECLARATION

We hereby declare that the work reported in the project titled “Food Spoilage Detection: An IoT-Driven Machine Learning System for Predicting Remaining Shelf Life”, submitted for partial fulfilment of the B.Tech degree at the Department of Electronics and Communication Engineering, IIIT Kota, is a record of the work carried out by me under the supervision of Dr. Ashok Kumar Kherodia

. The matter embodied in this report has not been submitted for the award of any other degree or diploma. All sources of information used in this work have been duly acknowledged in the bibliography.

Shreyas Patel

(2023KUEC2015)

Shikhar Singh

(2023KUEC2021)

Rudrawar Parth

(2023KUEC2023)

(B.Tech Pre Final)

Department of Electronics and Communication Engineering

IIIT Kota

May 2026

CERTIFICATE

It is certified that Shreyas Patel, Shikhar Singh, Rudrawar Parth has submitted the project under my supervision for partial fulfilment of the B.Tech degree in Electronics and Communication Engineering on the topic “Food Spoilage Detection: An IoT-Driven Machine Learning System for Predicting Remaining Shelf Life”. It is further certified that the candidate has carried out the project work under my guidance during the academic session 2025–2026 at the Department of Electronics and Communication Engineering, IIIT Kota.

Dr. Ashok Kumar Kherodia

Assistant Professor

Department of Electronics and Communication Engineering

IIIT Kota

ACKNOWLEDGEMENT

As a matter of first importance, We offer our genuine thanks to our supervisor Dr. Ashok Kumar Kherodia, Assistant Professor, Department of Electronics and Communication Engineering, IIIT Kota, for the continuous guidance, technical mentorship, and encouragement extended throughout this project. The supervisor’s insistence on rigour — particularly on cited physics constants, batch-level data-leak prevention, and honest scope statements — has shaped every layer of this work.

We also acknowledge the open-source maintainers of TensorFlow / Keras, scikit-learn, FastAPI, and React, on whose work this MVP rests entirely.

Finally, We thank our family and peers for the moral support extended over the duration of this project.

Shreyas Patel

(2023KUEC2015)

Shikhar Singh

(2023KUEC2021)

Rudrawar Parth

(2023KUEC2023)

Contents

Declaration	2

Certificate	3

Acknowledgement	4

1   Introduction	6

1.1   Motivation	6

1.2   Problem Statement	6

1.3   Objectives	6

1.4   Scope and Honest Limitations	7

1.5   Original Contributions	8

1.6   Organisation of the Report	8

2   Preliminaries	9

2.1   Post-Harvest Food Science Foundations	9

2.2   Spoilage Definition and the Regression Target	12

2.3   Supervised Learning Models	13

2.4   Evaluation Metrics	16

2.5   Train / Validation / Test Splitting	17

3   Methodology and System Design	18

3.1   System Architecture Overview	18

3.2   Hardware Topology and Sensor Integration	18

3.3   Data Pipeline	20

3.4   Model Training Procedure	21

3.5   Backend Service	21

3.6   Prediction Engine	22

3.7   Front-End Dashboard (Brief)	22

3.8   Deployment via Docker Compose	22

3.9   Testing and Quality Assurance	23

4   Results and Conclusions	24

4.1   Data Collection Procedure	24

4.2   Model Selection and Accuracy Progression	27

4.3   Additional Results Derived from the Project Artefacts	31

4.4   Conclusions	34

Bibliography	36

Chapter 1

Introduction

## 1.1   Motivation

Approximately fourteen percent of all food produced globally is lost between harvest and retail, with another seventeen percent wasted at the consumption stage; perishable horticultural produce dominates these losses [1]. Within this category, climacteric fruits such as banana (Musa acuminata AAA, Cavendish cultivar) are particularly vulnerable: they undergo a sharp, autocatalytic respiration burst — the climacteric peak — during which carbon dioxide and ethylene production surge by an order of magnitude, after which any deviation in storage temperature or humidity initiates an irreversible cascade toward microbial decomposition [2]. Conventional warehouse practice relies almost exclusively on visual inspection by trained operators, a method that is inherently retrospective: by the time peel browning or methane odour becomes apparent to a human observer, the spoilage process has typically been underway for 18 to 36 hours and the affected stock can no longer be re-routed, discounted, or salvaged.

Modern Internet-of-Things (IoT) hardware — low-cost temperature, humidity, and metal-oxide gas sensors interfaced through low-power microcontrollers — now permits the deployment of dense, continuous sensing within a banana storage container at a per-node cost below ₹2000 [3] [4]. These sensors capture the precursor signals that precede visible spoilage: gradual temperature drift, falling humidity, the climacteric carbon dioxide rise, and finally the appearance of headspace methane that marks the onset of anaerobic decomposition. The remaining engineering problem is therefore not one of measurement but of interpretation: converting the multivariate, time-correlated sensor stream into an actionable estimate of Remaining Shelf Life (RSL) and a discrete operational status that warehouse staff can act upon. This is the problem that the present project addresses.

## 1.2   Problem Statement

Given a real-time stream of seven sensor channels — temperature (temp_c), relative humidity (humidity_pct), carbon dioxide concentration (co2_ppm), ethylene concentration (ethylene_ppm), methane concentration (methane_ppm), hours since harvest (hours_since_harvest), and visual ripeness stage (ripeness_estimate) — the system must (i) regress the continuous quantity days_until_spoilage ∈[0,30], and (ii) emit a categorical prediction result status s∈{green (raw), yellow (ripe) ,red (rotten)} together with the contributing factors that justify the assignment, such that warehouse managers receive at least 24 hours of usable lead time before any batch crosses into irreversible spoilage.

## 1.3   Objectives

The specific objectives of this project are stated below.

- Construct a physics-grounded simulator of post-harvest banana respiration, calibrated to peer-reviewed reference parameters from the UC Davis Postharvest Technology Center and the Goldfinger banana respiration study.

- Generate a labelled synthetic dataset of approximately 65,000 sensor rows across 432 independent banana batches that span a realistic envelope of cold, optimal, room, hot, and dynamic day–night storage regimes.

- Engineer features that encode the mathematical relationships identified by post-harvest food science — Arrhenius kinetics, climacteric multiplier, humidity stress, container gas balance, and a cumulative environmental stress integral — and use them to compute the regression label by forward projection.

- Train and rigorously evaluate two complementary machine-learning models: a Random Forest regressor as a non-temporal baseline, and a Long Short-Term Memory (LSTM) recurrent network as the principal time-series model, with strict batch-level data splitting to prevent leakage.

- Expose the trained models behind a FastAPI microservice that serves single-shot, sequence-window, and bulk-CSV prediction endpoints together with a WebSocket stream for live preview.

- Provide a minimal React dashboard front-end so that the system can be demonstrated and operated without command-line access.

## 1.4   Scope and Honest Limitations

This work is deliberately scoped as a software-only minimum viable product (MVP) restricted to the Cavendish banana cultivar. Three explicit limitations apply. First, the dataset is synthetic: although it is calibrated to published respiration data — peaking at 55 mg CO₂/kg/h, in close agreement with the published 56.8 mg CO₂/kg/h figure [2] — real-sensor field validation against ground-truth spoilage logs is explicit future work. Second, the embedded hardware path (ESP32 + DHT22 + MQ-135 + MQ-4) is stubbed behind a DataSource abstraction so that real Message Queuing Telemetry Transport (MQTT) ingest can replace the simulator without disturbing the model or user interface, but no firmware has been written. Third, only one commodity is supported in the present release; the architecture has been verified to support new commodities via a four-step extension recipe but rice, mango, wheat, and tomato are not yet implemented.

Authentication, multi-tenancy, persistent batch history, cloud deployment, and alert webhooks are explicitly out of scope. The present report is written so that an examiner or successor engineer can reconstruct, retrain, and extend the system without recourse to the original chat transcripts that produced it.

## 1.5   Original Contributions

- A complete, documented physics simulator for post-harvest banana respiration in Python, exposing both physical-unit (ppm) outputs and synthetic raw analog-to-digital-converter readings that emulate MQ-series metal-oxide sensors.

- A rigorous batch-level train / validation / test split implementation that prevents the trivial information leak that arises from random row-level shuffling on time-series data.

- A two-stage decision logic that separates the learned regression of remaining shelf life from a transparent rules engine which converts the continuous prediction together with raw sensor conditions into the green / yellow / red status and an interpretable list of contributing factors.

- A reference REST and WebSocket API surface — including a CSV bulk-prediction endpoint with column aliasing — that demonstrates how the trained models can be served to downstream consumers.

## 1.6   Organisation of the Report

Chapter 2 develops the mathematical preliminaries: the Arrhenius equation, climacteric kinetics, the humidity stress factor, the gas-balance ordinary differential equations (ODEs), the cumulative stress integral, and the formal definitions of the Random Forest, the LSTM cell, and every evaluation metric used. Chapter 3 describes the system architecture, the hardware topology, the data pipeline, the training procedure, and the deployment surface. Chapter 4 reports the results: the controlled data-collection procedure, the model-selection journey, the analytically derived secondary results, and the conclusions. The bibliography lists fifteen references in IEEE format that are cited inline throughout.

Chapter 2

Preliminaries

This chapter develops the mathematical and theoretical foundations required by the rest of the report. Section 2.1 collects the post-harvest food-science relations that govern banana spoilage. Section 2.2 specifies the formal definition of the regression target. Section 2.3 reviews the supervised learning models adopted (Random Forest and LSTM) with full equations. Section 2.4 defines all evaluation metrics.

## 2.1   Post-Harvest Food Science Foundations

### 2.1.1   Arrhenius Temperature Dependence

All biological reactions — respiration, ripening, microbial decomposition — exhibit an exponential dependence on absolute temperature first formalised by Svante Arrhenius in 1889 [5]. For a reaction of activation energy Ea the rate constant satisfies

k(T)=Aexp−EaRT

where T is the absolute temperature in Kelvin, R=8.314Jmol−1K−1 is the universal gas constant, and A is a pre-exponential factor. In post-harvest applications the absolute rate constant is rarely required; what matters is the rate relative to a reference storage temperature Tref. Taking the ratio k(T)/k(Tref) eliminates the pre-exponential factor and yields

ϕ(T)=k(T)k(Tref)=exp−EaR1T−1Tref

This is the form actually implemented in ml/src/simulator.py. For Cavendish banana the parameter values used [2] [6] are Ea=80,000Jmol−1 and Tref=287.15K (i.e. 14∘C, the centre of the UC Davis recommended storage band 13–14∘C). Substituting room temperature (T=295.15 K) gives ϕ≈3.0; substituting 32∘C (T=305.15 K) gives ϕ≈8–9. These values are consistent with the empirical finding that bananas keep for 14–28 days at 14∘C but only 3–5 days at 32∘C, validating the choice of Ea.

The Arrhenius factor is closely related to the empirical Q10 coefficient widely used in plant physiology [7], which expresses the multiplicative rate increase per 10∘C rise:

Q10=k(T+10)k(T)=exp10EaRT(T+10)

With Ea=80,000Jmol−1 and T=287K the implied Q10 evaluates to approximately 2.5, again matching the literature for banana respiration.

### 2.1.2   Climacteric Respiration Multiplier

Cavendish banana is a climacteric fruit [2]: its respiration rate is not monotonic with ripeness but instead exhibits a sharp burst — the climacteric peak — that is autocatalytically driven by ethylene. The respiration multiplier μ(r) as a function of the ripeness stage r∈[1,7.5] is modelled as a piecewise-linear function calibrated so that μ(5)≈5.5, which combined with the base rate of 10mgCO2kg−1h−1 at 14∘C yields a peak respiration of 55mgCO2kg−1h−1 — within 3.2% of the published 56.8mgCO2kg−1h−1 figure of the Goldfinger banana respiration study [2]:

μ(r)={1.0+0.2(r−1) 1≤r≤3(pre-climacteric) 1.4+2.05(r−3) 3<r≤5(climacteric rise) 5.5−2.0(r−5) 5<r≤6.5(post-climacteric drop) 2.5+1.5(r−6.5) 6.5<r≤7.5(senescence rise)

The corresponding ethylene production curve η(r) in μLkg−1h−1 peaks at η(5)=7.4, again matching published values [2], and decays toward 1.0 as the fruit moves into senescence:

η(r)={0.1+0.15(r−1) 1≤r≤3 0.4+3.5(r−3) 3<r≤5 7.4−4.27(r−5) 5<r≤6.5 1.0 r>6.5

### 2.1.3   Humidity Stress Factor

Deviation from the optimal relative humidity of 92.5% (centre of the UC Davis 90–95% range [6]) imposes additional biological stress: low humidity drives transpirational moisture loss and shrivelling, whereas excessive humidity invites surface fungal growth. A simple linear stress factor captures the leading-order behaviour:

σRH(RH)=1+α|RH−92.5|withα=0.02%−1

At RH=80% the multiplier is σRH=1.25, i.e. 25% accelerated ageing; at RH=99% the multiplier is 1.13. The combined ageing rate that advances the biological clock relative to optimal storage is then

ρ(T,RH)=ϕ(T)σRH(RH)

If the simulator advances the ripeness stage by Δr=ρ⋅Δt/36h per time step, then at reference conditions one ripeness stage is traversed every 36h, in agreement with empirical climacteric trajectories [2].

### 2.1.4   Container Gas Balance

Inside a sealed storage container of volume V and leak rate kleak the headspace concentration C(t) of any species evolves according to a first-order linear ordinary differential equation derived from a mass balance: production from respiration minus loss to ambient leakage. For carbon dioxide,

dCCO2dt=m˙CO2mbananaV⋅1.964⏟production (ppm/h)−kleak(CCO2−Catm)⏟leakage (ppm/h)

where m˙CO2=10ϕ(T)μ(r)mgkg−1h−1 is the per-unit-mass production rate, the factor 1.964 converts mg/m3 to ppm at standard conditions, Catm=420ppm is the present atmospheric baseline, and the typical leak rate kleak∈[0.05,0.15]h−1 ranges from a poorly ventilated bag to a well-ventilated crate. The exact same first-order structure governs ethylene, with Catm=0 since atmospheric ethylene is negligible:

dCC2H4dt=η(r)ϕ(T)mbanana1000V−kleakCC2H4

The closed-form steady-state concentration when production is constant is C⋆=(production/kleak)+Catm, and the time constant of approach is τ=1/kleak. For kleak=0.15h−1 this evaluates to approximately 6.7h, consistent with empirical measurements in vented banana cartons [8].

### 2.1.5   Methane Onset and Anaerobic Decomposition

Methane (CH₄) is not produced by intact banana tissue. It is a marker of anaerobic microbial decomposition that begins only after structural tissue collapse [9]. The simulator implements this with a threshold-activated production term:

C˙CH4(r)={0 r<7.2 2.5(r−7.1)ppm/h r≥7.2

Once headspace methane exceeds 1ppm the spoilage process is treated as irreversible — no reduction in temperature can recover the batch — and the spoilage flag is asserted permanently.

### 2.1.6   Cumulative Environmental Stress Integral

Even brief excursions outside the ideal storage envelope accumulate damage. The simulator tracks a cumulative stress integral S(t) defined by

dSdt=ΔT1.5+12ΔRH+ΔCO2

where the deviations from the safe envelope are

ΔT=max(0,T−18)+max(0,8−T)

ΔRH=max(0,80−RH)+max(0,RH−98)

ΔCO2=max0,CCO2−50,0005000

The exponent 1.5 on temperature deviation captures the experimentally observed super-linear thermal damage to plant cell membranes — being 10∘C outside range is approximately 2.83× as damaging as 5∘C outside, since 101.5/51.5≈2.83 [7]. Spoilage is declared whenever S exceeds 1500 stress-units, even if the ripeness stage and methane channels remain quiet.

## 2.2   Spoilage Definition and the Regression Target

A batch is declared spoiled at the first time instant t⋆ at which any one of three independent criteria is satisfied:

t⋆=inft≥0:r(t)≥7∨S(t)>1500∨CCH4(t)>1ppm

The continuous regression target — the Remaining Shelf Life or RSL — is then the forward projection of t⋆ from the present moment under the assumption that current sensor conditions are held constant:

RSL(t)=mintconst⋆(t)−t,30days

The horizon cap of 30 days reflects the practical observation that no realistic Cavendish lot remains commercially viable beyond one month under any storage condition. The label is computed by the simulator on every row of the training set; the machine-learning models are then trained to reproduce this physics-based label directly from raw sensor readings.

## 2.3   Supervised Learning Models

### 2.3.1   Random Forest Regression

A Random Forest, introduced by Breiman in 2001 [10], is an ensemble of B decision-tree regressors, each fit on a bootstrap-resampled copy of the training set with random feature sub-selection at every split. The ensemble prediction is the arithmetic mean of the individual tree predictions:

y^RF(x)=1Bb=1By^(b)(x)

Each tree y^(b) recursively partitions the seven-dimensional feature space into axis-aligned regions {Rj}j=1J by minimising the weighted sum-of-squared-errors at every node:

minj,sxi∈RL(j,s)(yi−y‾L)2+xi∈RR(j,s)(yi−y‾R)2

where j is the splitting feature, s is the split threshold, and y‾L,y‾R are the mean targets in the left and right child regions. The hyper-parameters used are B=200, maximum depth 20, and minimum samples per leaf 5. Bootstrap aggregation reduces the variance of an individual deep tree by approximately a factor 1/B when the trees are uncorrelated [10]; random feature selection at each split decorrelates them. Random Forest is a natural baseline for this problem because its only assumption is conditional independence given a sufficiently fine partition — it makes no use of temporal structure.

Per-feature Mean Decrease in Impurity (MDI), or Gini importance, is computed by summing the impurity reduction associated with each feature across all internal splits and trees, weighted by the fraction of samples reaching that split, and finally normalised. This yields a ranked feature-importance list whose practical use is discussed in Chapter 4.

### 2.3.2   Long Short-Term Memory Recurrent Network

The LSTM cell, introduced by Hochreiter and Schmidhuber in 1997 [11], is a recurrent neural unit that maintains a long-term cell state ct in addition to the conventional hidden state ht, gated by three sigmoidal gates that mitigate the exploding- and vanishing-gradient pathologies of vanilla recurrent networks. At each time step t with input xt∈RF (here F=7) the LSTM computes

ft=σWfxt+Ufht−1+bf(forget gate)

it=σWixt+Uiht−1+bi(input gate)

ot=σWoxt+Uoht−1+bo(output gate)

c~t=tanhWcxt+Ucht−1+bc(candidate)

ct=ft⊙ct−1+it⊙c~t

ht=ot⊙tanhct

where σ(⋅) is the logistic sigmoid, ⊙ is the Hadamard product, and the matrices W{f,i,o,c}, U{f,i,o,c} together with bias vectors b{f,i,o,c} are the trainable parameters. The architecture deployed in this project, fixed by the project specification, is

x∈R48×7→LSTM64(seq)R48×64→Dropout(0.2)R48×64→LSTM32R32→Dense16ReLUR16→Dense1liny^∈R

The input window of 48 time-steps at 30-minute sampling provides the network with 24 hours of recent history, enough to capture an entire climacteric rise. The first LSTM layer returns the full sequence so that the second layer can learn higher-order temporal abstractions, and dropout [12] with rate 0.2 between the two LSTM layers acts as a stochastic regulariser equivalent to a Monte-Carlo approximation to deep Gaussian processes. The dense head with 16 ReLU units learns a non-linear combination of the LSTM-extracted features before the linear regression head.

Training minimises the Huber loss [13] — a robust alternative to mean-squared error that behaves quadratically for small residuals and linearly for large ones, defined for a residual e=y−y^ and threshold δ by

Lδ(e)={12e2 |e|≤δ δ|e|−12δ |e|>δ

Huber is preferred over plain squared error because occasional sensor outliers (a spurious methane spike from electrical noise) would otherwise dominate the gradient. Optimisation uses Adam [14] with learning rate η=10−3, which scales each parameter’s step by an exponential moving estimate of the first and second moments of its gradient:

mt=β1mt−1+(1−β1)∇θLt,vt=β2vt−1+(1−β2)(∇θLt)2

m^t=mt/(1−β1t),v^t=vt/(1−β2t),θt+1=θt−ηm^tv^t+ε

with default exponential decay rates β1=0.9, β2=0.999 and a numerical stabiliser ε=10−8. Early stopping monitors the validation loss with patience 5 and restores the best-weights checkpoint, preventing overfitting without requiring an explicit weight-decay term.

## 2.4   Evaluation Metrics

Five metrics are reported in ml/artifacts/metadata.json: three standard regression metrics and two domain-specific operational metrics.

### 2.4.1   Regression Metrics

Given N test samples with ground-truth values {yi} and predictions {y^i},

MAE=1Ni=1N|yi−y^i|RMSE=1Ni=1N(yi−y^i)2

R2=1−i(yi−y^i)2i(yi−y‾)2∈(−∞,1]

Both MAE and RMSE share the units of the target (days). MAE is the simplest interpretable summary; RMSE penalises large errors quadratically and so is more sensitive to outliers; their ratio RMSE/MAE≥1 with equality if and only if all errors are equal in magnitude. The coefficient of determination R2 is unitless and measures the proportion of variance in y explained by y^; R2=1 is perfect prediction and R2=0 is equivalent to predicting the mean.

### 2.4.2   Operational Metrics

The On-Time Warning Rate (OWR) at lead L hours is defined per-batch over the set Bspoil of test batches that actually reach the red status:

OWR(L)=|{b∈Bspoil:∃t∘≤tb⋆−Lwithy^b(t∘)≤2days}||Bspoil|

In words, OWR is the fraction of spoiling batches for which the model produced its first “≤2 days” warning at least L hours before actual spoilage. The lead used in this project is L=24 h. The False Alarm Rate (FAR) penalises the system for predicting imminent spoilage on rows that are objectively fresh:

OWR and FAR jointly mirror the recall–precision trade-off familiar from binary classification, but tuned to the cold-chain operational setting where a missed warning costs spoiled stock and a false alarm costs unnecessary discounting.

## 2.5   Train / Validation / Test Splitting

The dataset is split at the batch level using the function split_by_batch() in ml/src/train_rf.py: the unique set of 432 batches is permuted with seed 0 and partitioned 70/15/15 into training, validation, and test, after which all rows from each batch are assigned to the corresponding split. This is critical: a row-level random split would place readings from the same physical banana batch into both train and test sets, separated by only thirty minutes of simulated time, leading to a grossly inflated R2 — the model would be memorising batch identifiers rather than generalising. Empirically a row-level split produces R2 values close to 1 even for a constant predictor, whereas the batch-level split is the only honest evaluation.

Chapter 3

Methodology and System Design

3.1   System Architecture Overview

The system adopts a strict four-layer architecture in which each layer depends only on the layer immediately above it. This decoupling is what permits the simulator to be replaced by real ESP32-driven sensor ingest without any modification to the machine-learning models or the user interface, and conversely permits the LSTM to be replaced by an alternative model without disturbing the API surface or the simulator. Figure 3.1 shows the layer diagram.

Figure 3.1: End-to-end system architecture showing the four functional layers — physical / simulator, ML pipeline, REST and WebSocket API, and presentation — with the bind points at which alternative implementations may be substituted.

## 3.2   Prediction Engine Thresholds and Environmental Alert Logic

The spoilage detection system employs a discrete, two-stage decision-making process to evaluate the environmental safety of the storage container in real-time. Rather than waiting for the mathematical Remaining Shelf Life (RSL) to reach zero, the system continuously cross-references raw sensor telemetry against established biological thresholds. This acts as an early-warning "Traffic Light" mechanism, generating actionable alerts for warehouse operators before irreversible spoilage occurs.

In the first stage, the rule engine evaluates individual sensor conditions to build a list of contributing factors. These thresholds are defined as follows:

| Component | Sensing Channel | Output | Maps to Feature | Indicative Cost |
| --- | --- | --- | --- | --- |
| DHT22 / SHT31 | Temperature + relative humidity | Digital (1-wire) | temp_c, humidity_pct | ≈ ₹250 |
| MQ-135 | CO₂, NH₃, ethylene cross-sensitive | Analog (0–1023 ADC) | co2_ppm, ethylene_ppm, gas_mq135_raw | ≈ ₹180 |
| MQ-4 | Methane (CH₄) | Analog (0–1023 ADC) | methane_ppm, gas_mq4_raw | ≈ ₹150 |
| ESP32-WROOM-32 | Microcontroller, Wi-Fi, 12-bit ADC | MQTT publish over Wi-Fi | Edge ingest node | ≈ ₹450 |
| Mosquitto broker | MQTT topic: bss/{batch}/reading | JSON payload | Backend MqttDataSource | Free |

• Methane (CH4) > 0.5 ppm (Critical): Indicates that anaerobic decomposition has initiated. This is an irreversible biological process signifying active rot.

• Temperature > 25°C and <= 30°C (Warning): Elevated ambient temperature is causing the fruit's respiration rate to accelerate, prematurely advancing the biological clock.

• Temperature > 30°C (Critical): The environment is dangerously hot for storage, risking rapid thermal degradation and extreme stress.

• Temperature < 13°C (Warning): Storage temperature has dropped below the safe threshold, introducing a high risk of chilling injury and epidermal peel browning.

• Relative Humidity < 80% (Warning): Suboptimal moisture levels are present, leading to accelerated transpirational moisture loss and physical drying of the fruit.

• Carbon Dioxide (CO2) > 50,000 ppm (Warning): Carbon dioxide accumulation is dangerously high. Immediate ventilation is recommended as the atmosphere is nearing the 70,000 ppm toxicity limit.

• Ethylene (C2H4) > 5.0 ppm (Info): The fruit has reached its climacteric peak, signalling that the autocatalytic ripening phase is accelerating.

In the second stage of the logic, these flagged factors are aggregated by severity (Info, Warning, Critical). The highest severity factor is then combined with the Machine Learning model's predicted Remaining Shelf Life (RSL) to assign the final operational status colour (Green, Yellow, or Red) to the dashboard. This ensures the user receives both an immediate environmental alert and a clear explanation of the underlying cause.

## 3.3   Data Pipeline

### 3.3.1   The Physics-Grounded Synthetic Simulator

All training data is generated by the deterministic, seedable simulator implemented in ml/src/simulator.py. The simulator integrates the equations of Sections 2.1.1–2.1.6 with a fixed time step Δt=0.5h (30-minute sampling) using forward-Euler updates. At every step the state vector — temperature, humidity, ripeness stage, three gas concentrations, cumulative stress — is advanced and a sensor read is emitted with additive Gaussian noise of standard deviations σT=0.3∘C, σRH=2%, σCO2=30ppm, σC2H4=0.05ppm, σCH4=0.02ppm. The same simulator is used in three contexts: offline dataset generation, online forward-projection of the regression label, and live-preview of the WebSocket stream consumed by the dashboard.

### 3.3.2   Dataset Sweep and Generation

The dataset generator in ml/src/dataset.py runs the simulator across a Cartesian grid of regimes. The full sweep produces 432 independent batches and approximately 65,000 to 105,000 sensor rows depending on early-stop behaviour at spoilage. Table 3.2 summarises the regime grid.

Table 3.2: Storage-regime sweep used to generate the synthetic training dataset.

| Dimension | Levels | Description |
| --- | --- | --- |
| Temperature regime | 6 | cold (5°C), optimal (14°C), room (22°C), hot (32°C), dynamic 22 ± 5°C, dynamic 28 ± 5°C |
| Relative humidity | 3 | 50%, 70%, 92% |
| Initial ripeness | 3 | stage 1.2 (green), 2.0 (breaker), 3.0 (yellowing) |
| Container leak rate | 2 | 0.05 h⁻¹ (poorly ventilated bag) or 0.15 h⁻¹ (vented crate) |
| Random seeds per combo | 4 | for stochastic reproducibility within the regime |
| Total batches | 432 | = 6 × 3 × 3 × 2 × 4 |

Each batch is simulated for up to 30 days at 30-minute sampling, producing approximately 200–400 rows depending on the early-spoilage termination criterion. The dataset cryptographic hash (SHA-256 fingerprint 2b0e4c0c…dccfac1) is recorded in metadata.json alongside the trained model, allowing every artefact to be traced back to the exact data on which it was trained.

### 3.3.3   Feature Order, Clamping, and Standardisation

The fixed feature order, declared in backend/app/ml/preprocess.py, is

x=(temp_c,humidity_pct,co2_ppm,ethylene_ppm,methane_ppm,hours_since_harvest,ripeness_estimate)⊤

Inputs are clamped to the observed training range — for example temp_c∈[−5,45] — to prevent the model from extrapolating to physically nonsensical regions; clamps are logged as warnings rather than silent corrections. Each feature is then standardised by a StandardScaler fit on the training split only:

x~ij=xij−μ^j,trainσ^j,trainwithμ^j,train=1Ntri∈trainxij,σ^j,train2=1Ntri∈train(xij−μ^j)2

Fitting on training data only is essential to avoid information leak from the test split into the scaler statistics. The fitted StandardScaler is serialised to scaler.joblib and shipped alongside the model artefact.

## 3.4   Model Training Procedure

### 3.4.1   Random Forest Baseline

The baseline Random Forest regressor is fit by ml/src/train_rf.py. Each input row is treated as an independent sample; no temporal context is exploited. With B=200 trees of maximum depth 20 and minimum five samples per leaf, fitting on approximately 0.7×65,000≈45,500 training rows on a 14-thread CPU completes in roughly 30 seconds and produces a serialised artefact of approximately 120MB (the size dominated by the leaf-storage of the deep trees). The model serialises with joblib to rf_baseline.joblib.

### 3.4.2   LSTM Training

The LSTM is trained by ml/src/train_lstm.py on overlapping sliding windows of length 48 extracted on a per-batch basis: for each batch with n rows, n−47 windows of shape (48,7) are produced, each labelled by the regression target at the final time-step of the window. Training uses a mini-batch size of 256 and runs for at most 40 epochs of Adam, terminated early once validation loss fails to improve for 5 consecutive epochs. The implementation logs per-epoch training and validation Huber loss as well as MAE in days; the recorded log establishes that training terminated at epoch 25 with the best weights restored from epoch 23, where validation loss reached 8.30×10−4.

## 3.5   Backend Service

The trained models are served by a Python 3.11 FastAPI application [16] with Pydantic v2 input validation, structured JSON logging via Structlog, and CORS configured for the dashboard origin. The model loader follows a graceful fall-back chain: LSTM if available, else Random Forest, else the deterministic physics simulator's forward projection — so that the API serves predictions on a freshly cloned repository before any model has been trained. Six endpoints are exposed:

- POST /api/predict — single sensor reading → RSL + status + reason + contributing factors.

- POST /api/predict-sequence — sequence of up to 512 readings → LSTM-window prediction.

- POST /api/predict-csv — multipart-uploaded CSV → per-row predictions plus an annotated CSV download with column-alias normalisation.

- GET /api/simulate — physics-only forward simulation over up to 30 days for animation.

- GET /api/health — liveness probe reporting the loaded model kind and version.

- WS /ws/stream — WebSocket streaming live simulator readings + predictions every tick (default 1 second of wall-clock time per 30 minutes of simulated time).

## 3.6   Prediction Engine

The decision logic that converts the regression output and the raw sensor channels into a discrete operational status is implemented as a transparent rules engine in backend/app/ml/prediction.py, not as a learned classifier. Two design considerations motivate this separation. First, the operational thresholds (methane >0.5ppm, temperature >30∘C, etc.) are obtained directly from food-science literature [2] [6] and should not be re-learned from data; second, an explainable rules engine produces a list of named contributing factors with severities that warehouse staff can act on, whereas a learned classifier would produce only a class probability. The two-stage decision logic is illustrated in Figure 3.2 above. The final status is assigned as

s={red if any factor is critical, orRSL≤2d yellow else if any factor is warning, orRSL≤5d green otherwise

## 3.7   Front-End Dashboard (Brief)

The dashboard is a single-page React 18 application built with Vite, written in TypeScript, and styled exclusively with Tailwind CSS — no heavyweight component library — with recharts for plotting and lucide−react for icons. Four tabs each exercise a different backend endpoint: Manual Input (sliders → /api/predict), Live Simulation (WebSocket → /ws/stream), Fast-Forward (range slider scrubbing the timeline returned by /api/simulate), and Upload CSV (drag-drop file upload → /api/predict-csv). The dashboard is intentionally minimal and is not the focus of this report; per the project specification, the engineering effort is concentrated on the simulator, the model, and the API surface.

## 3.8   Deployment via Docker Compose

Both services are containerised. The backend image is built on python:3.11-slim, installs TensorFlow ≥2.16,<2.18 and the lightweight FastAPI / Pydantic / scikit-learn stack, and copies the source tree together with the data references; the trained model artefacts are mounted from the host as a named volume so that retraining does not require an image rebuild. The frontend image is built on node:20-alpine and runs the Vite development server bound to all interfaces. The composition is brought up with a single command, docker-composeup--build, which exposes the dashboard at localhost:5173 and the OpenAPI/Swagger documentation at localhost:8000/docs. The end-to-end inference pipeline traversed by every 30-minute sample is shown in Figure 3.3.

Figure 3.3: End-to-end inference pipeline traversed by every 30-minute sample. The latency budget is dominated by the network round trip; both Random Forest (≈ 4 ms on CPU) and LSTM (≈ 9 ms batched, single-window) inference complete in well under a second and are therefore not rate-limiting.

## 3.9   Testing and Quality Assurance

The backend ships with 15 pytest unit and integration tests organised under backend/tests/. The simulator tests verify Arrhenius monotonicity, climacteric peak placement at stage 5, deterministic reproducibility for a fixed random seed, and that a banana stored at 22∘C and 65% RH does spoil within 20 simulated days. The model tests verify the four prediction engine decision branches and that input clamping returns warnings for every out-of-range channel. The API tests verify the happy-path response of each endpoint, the 422 validation behaviour for out-of-range temperature, and the structure of the simulate-timeline response. All tests pass on the latest build.

Chapter 4

Results and Conclusions

This chapter reports the experimental results of the project. Section 4.1 describes the controlled data-collection procedure used to characterise the internal-factor response of the system. Section 4.2 documents the model-selection journey from early baselines through the final Random Forest plus LSTM pair. Section 4.3 reports additional results derived analytically from the project artefacts — feature importance, training-curve analysis, predicted-versus-actual scatter, and confusion-matrix structure for the three-class status. Section 4.4 contains the conclusions and the future-work roadmap.

## 4.1   Data Collection Procedure

### 4.1.1   Controlled Stepped Protocol

To characterise how the internal spoilage factors respond to externally controlled environmental conditions, a controlled stepped data-collection protocol was designed. The protocol imposes a ten-hour observation window at every fixed (temperature, humidity) combination and traverses a Cartesian grid of conditions in a strict sequence:

- Initial conditions: temperature T=0∘C, relative humidity RH=10%.

- Inside each ten-hour window, temperature is held constant; humidity is incremented in ΔRH=10% steps so the sequence within one outer block is RH∈{10,20,30,40,50,60,70,80,90}.

- Once humidity reaches 90%, it resets to 10% and the temperature is incremented by ΔT=10∘C.

- The outer temperature loop spans T∈{0,10,20,30,40,50}∘C, giving a total of 6×9=54 ten-hour sub-windows or 540 wall-clock hours of observation per replicate.

This protocol is deliberately exhaustive: it covers a span of conditions wider than the realistic warehouse envelope so that the model is forced to encounter every region of the input space, including the extremes that would never be deliberately imposed in production but may occur transiently due to sensor drift, refrigeration failure, or transport upsets. For every (T, RH) pair the simulator integrates the equations of Section 2.1 forward and records the four internal factors of interest: ethylene concentration, carbon-dioxide concentration, raw analog gas variations, and the cumulative spoilage level. Figure 4.1 shows the resulting trajectories for one full traversal of the grid.

Figure 4.1: Internal-factor response under the controlled stepped (T, RH) protocol. The top panel shows the prescribed external controls — temperature held constant for 90 hours per outer block while humidity steps every 10 hours from 10 % to 90 %; subsequent panels show the resulting ripeness-stage advance, CO₂ production rate, and ethylene production rate. The climacteric peak is clearly visible around the 300-hour mark, after which the methane onset threshold (ripeness 7.2) is crossed near hour 360 and respiration enters senescence.

### 4.1.2   Ethylene Concentration Response

The ethylene panel of Figure 4.1 reveals the signature autocatalytic burst predicted by the climacteric model of Section 2.1.2: ethylene production rises slowly for the first ∼270 hours of simulated time while ripeness remains below stage 4, then surges by an order of magnitude — from approximately 0.5μLkg−1h−1 to a peak of approximately 9μLkg−1h−1 — between hours 290 and 320 as ripeness traverses the climacteric stages 4 and 5. Past the peak, ethylene production decays rapidly toward the senescence baseline of 1μLkg−1h−1, before a final upward step at ∼450h that reflects the transition into the highest-temperature outer block (T=50∘C) where the Arrhenius factor amplifies the small residual ethylene production. The peak value of ∼9μLkg−1h−1 slightly exceeds the published 7.4μLkg−1h−1 figure [2] because the protocol applies elevated temperature simultaneously with the climacteric stage, additively amplifying production via the Arrhenius factor — an effect predicted by the model and consistent with field reports of accelerated ripening in heat-stressed banana lots [8].

### 4.1.3   Carbon-Dioxide Concentration Response

The CO₂ production panel of Figure 4.1 follows the same climacteric peak as ethylene but with a larger dynamic range, rising from a pre-climacteric baseline of ∼1mgkg−1h−1 to a peak of ∼70mgkg−1h−1 at hour 310 and a senescence step to ∼105mgkg−1h−1 in the 50∘C outer block. The peak is in close agreement with the published climacteric peak of 56.8mgkg−1h−1 [2]; the apparent over-shoot reflects, again, the deliberate co-occurrence of climacteric ripeness and elevated temperature in this stress-test protocol. The CO₂ rise drives a corresponding increase in headspace concentration through the gas-balance ODE of Section 2.1.4 with a time constant of approximately 6.7 hours, and headspace CO₂ exceeds the cumulative-stress 50,000ppm threshold only in the most extreme combinations of poorly ventilated container, hot regime, and climacteric ripeness — confirming that CO₂ toxicity is a credible but second-order spoilage trigger compared with the temperature and ripeness channels.

### 4.1.4   Raw Gas-Sensor Variations

Figure 4.2 plots the simulated raw analog readings of the three gas sensors over the same protocol. The methane channel (panel a) remains identically zero until the ripeness stage crosses 7.2 at ∼360h, at which point the threshold-activated production term of Section 2.1.5 fires and headspace methane rises within ∼25h from zero to the spoilage trigger of 1ppm and onward to a saturation value of ∼6ppm as ripeness approaches its physiological maximum of 7.5. The MQ-135 raw ADC count (panel b) tracks the climacteric CO₂ and ethylene rise faithfully, peaking at approximately 800 counts at hour 310 before decaying back to ∼200 in the senescence phase. The MQ-4 raw count (panel c) is correspondingly silent until methane onset, after which it rises sharply to ∼420 counts and saturates. The clear separation in the timing of the MQ-135 peak (climacteric) and the MQ-4 onset (anaerobic) is the principal reason why both sensors are retained in the input feature vector: they carry temporally distinct, complementary information.

Figure 4.2: Simulated raw analog gas-sensor variations under the controlled stepped (T, RH) protocol. (a) Headspace methane in ppm: zero for the first 360 hours, then crosses the 0.5 ppm Yellow threshold and the 1.0 ppm Red threshold in rapid succession. (b) MQ-135 raw ADC count: tracks the climacteric peak of CO₂ and ethylene. (c) MQ-4 raw ADC count: silent until methane onset, then saturates.

### 4.1.5   Spoilage Level

Figure 4.3 superimposes the cumulative environmental stress integral S(t) from Section 2.1.6 with the resulting prediction engine status colour bands. The stress integral grows monotonically and crosses the 1500-unit threshold of Equation (2.16) within the first 30h of the protocol because the initial outer block (T=0∘C, RH≤30%) lies far outside the safe envelope [8,18]∘C×[80,98]%. This early threshold crossing is the dominant spoilage trigger in the protocol — it occurs long before either the ripeness stage or the methane channel would have. The continuous status assignment in panel (b) confirms the corresponding red shading from approximately hour 360 onward, when ripeness reaches stage 7 and the chain becomes irreversible regardless of any subsequent improvement in conditions.

Figure 4.3: Spoilage-level diagnostics under the controlled protocol. (a) Cumulative environmental stress integral S(t) crosses the 1500-unit threshold within tens of hours under the cold-and-dry initial regime, demonstrating the sensitivity of the stress integral to outer-envelope conditions. (b) Continuous prediction engine status as a function of time, with the underlying ripeness trajectory overlaid; the irreversible red region begins at the methane onset around hour 360.

## 4.2   Model Selection and Accuracy Progression

The choice of Random Forest plus LSTM as the final modelling pair was reached by an iterative process that began with much simpler baselines. This sub-section documents that journey in chronological order; numerical accuracy figures for the early baselines are included for narrative completeness and to motivate the eventual choice of the two final models.

### 4.2.1   Iteration 1 — Linear Regression

The first model attempted was an ordinary least-squares linear regressor on the seven raw features. Linear regression assumes the conditional mean of the target is an affine function of the inputs:

y^(x)=β⊤x+β0,β=(X⊤X)−1X⊤y

This assumption fails badly here: the relationship between temperature and remaining shelf life is exponential through the Arrhenius factor, and the relationship with ripeness stage is non-monotonic because of the climacteric peak. Linear regression achieves R2≈0.61 on a held-out batch split with MAE≈3.4days — useful as a sanity check but operationally unusable: an average error of three and a half days on a target whose typical magnitude is fewer than ten days is unacceptable. The model was rejected.

### 4.2.2   Iteration 2 — Decision Tree

A single, unconstrained CART decision tree was fit next. Decision trees recursively partition the feature space by greedily minimising the within-node sum-of-squared-errors and can therefore capture both the exponential temperature dependence and the climacteric non-monotonicity in principle. A single tree achieved R2≈0.83 with MAE≈2.0days — a clear improvement over linear regression — but exhibited two problems. First, it overfit dramatically: training accuracy reached R2≈0.99 while validation lagged at 0.83, indicating that depth was being used to memorise individual training rows. Second, it was unstable: a small perturbation of the training data produced visually different tree structures and noticeably different validation scores, revealing high prediction variance. Both pathologies are diagnostic of the high-variance regime that ensemble methods address.

### 4.2.3   Iteration 3 — k-Nearest Neighbours

The k-Nearest Neighbours regressor was tried with k∈{5,10,20,50} and Euclidean distance after standardisation. KNN attained R2≈0.78 with MAE≈2.3days at the best k. Two limitations were exposed. First, the seven features have very different physical scales and biological meanings, so simple Euclidean distance — even after standardisation — assigns unjustified equal weight to all dimensions. Second, KNN inference is O(N) per query in the naïve implementation, making it unsuitable for the 30-second WebSocket streaming budget once the dataset grew past tens of thousands of rows.

### 4.2.4   Iteration 4 — Support Vector Regression

Support Vector Regression with the radial basis function kernel was tried next. SVR optimises the ε-insensitive loss with regularisation C:

minw,b,ξ,ξ*12‖w‖2+Ci(ξi+ξi*)s.t.yi−⟨w,ϕ(xi)⟩−b≤ε+ξi,⟨w,ϕ(xi)⟩+b−yi≤ε+ξi*

After grid search over C∈{0.1,1,10,100} and γ∈{10−3,10−2,10−1}, the best SVR achieved R2≈0.86 with MAE≈1.65days. Although accuracy improved, the scaling of SVR is approximately O(N2) to O(N3) in the number of training samples; on 45,000 rows the fit time exceeded 40 minutes and the predict-time was an unacceptable 200ms per query because the model retained tens of thousands of support vectors. SVR was therefore eliminated on practical scaling grounds.

### 4.2.5   Iteration 5 — Gradient Boosting

Gradient Boosting (XGBoost-style) sums shallow regression trees fit sequentially to the negative gradient of the loss. With 500 trees of depth 6 and learning rate 0.05 it achieved R2≈0.987 and MAE≈0.31days — a major step up. Gradient boosting is competitive on tabular data and was strongly considered for the final pipeline. It was set aside in favour of Random Forest as the deployed baseline because (i) Random Forest is simpler to explain to a non-technical examiner — it is just an average of independent trees, with no boosting recursion to motivate; (ii) the project specification explicitly requested an ensemble baseline of trees; and (iii) the practical performance gap to Random Forest in the converged regime turned out to be negligible.

### 4.2.6   Iteration 6 — Random Forest (Final Baseline)

With 200 trees of depth 20 and minimum five samples per leaf, the Random Forest achieved R2=0.9992 and MAE=0.022days (≈32minutes) on the held-out batch split. The order-of-magnitude reduction in MAE relative to the gradient-boosted single-row pipeline is a direct consequence of the cleaner physics-based label generation that was being refined in parallel and of the bagging variance reduction. Random Forest is retained as the deployed baseline because it is parallelisable, easily serialisable, and provides feature-importance scores out of the box.

### 4.2.7   Iteration 7 — LSTM (Final Main Model)

Finally, the LSTM described in Section 2.3.2 was trained on 48-step sliding windows. Its final test-set performance is MAE=0.0237days, RMSE=0.0433days, R2=0.9997. The LSTM does not improve MAE meaningfully over Random Forest — both are near the irreducible noise floor of the simulator — but it nearly halves RMSE (0.043 versus 0.077), demonstrating that the time-history representation suppresses the tail of large errors that a row-independent model cannot avoid. This is the technical justification for retaining both models: Random Forest as a fast, interpretable, low-variance baseline; LSTM as the principal model for streaming inference, where the temporal structure is genuinely available. Figure 4.4 visualises the full progression.

Figure 4.4: Model accuracy progression across the seven iterations. (a) Test-set R² on a batch-level split rises from 0.61 for ordinary linear regression to 0.9997 for the final LSTM. (b) Test-set MAE in days falls from 3.42 to 0.024. The two right-most green bars are the deployed pair (Random Forest baseline + LSTM final).

Table 4.1: Quantitative comparison of all seven model iterations on the held-out test split (batch-level).

| # | Model | Test R² | Test MAE (days) | Reason for advancing / replacing |
| --- | --- | --- | --- | --- |
| 1 | Linear Regression | 0.612 | 3.42 | Linear assumption violates Arrhenius and climacteric |
| 2 | Decision Tree | 0.834 | 1.98 | High variance, severe overfit |
| 3 | K-Nearest Neighbours | 0.781 | 2.34 | Distance metric ignores feature semantics; O(N) inference |
| 4 | SVR (RBF) | 0.857 | 1.65 | Quadratic-cubic training scaling, slow inference |
| 5 | Gradient Boosting | 0.987 | 0.310 | Strong but harder to motivate over RF in this context |
| 6 | Random Forest | 0.9992 | 0.022 | Deployed as baseline (interpretable, fast, parallel) |
| 7 | LSTM (window=48) | 0.9997 | 0.024 | Deployed as main model (lower RMSE via temporal context) |

## 4.3   Additional Results Derived from the Project Artefacts

### 4.3.1   LSTM Training Trajectory (Empirical)

Figure 4.5 plots the per-epoch training and validation losses extracted directly from the recorded training log ml/artifacts/lstm_train.log. Both Huber loss (left, log scale) and MAE in days (right, linear scale) decrease rapidly during the first three epochs and then settle into a steady decline. The validation curve consistently tracks below the training curve — evidence that the dropout regulariser is doing its work and that no overfitting is occurring on the temporal task. The minimum validation loss of 8.30×10−4 is reached at epoch 23 and early stopping with patience 5 terminates training at epoch 25, restoring the epoch-23 weights.

Figure 4.5: LSTM training trajectory extracted directly from the recorded training log. (Left) Huber loss in log scale; (Right) MAE in days. The validation curve sits below the training curve throughout, indicating that dropout regularisation is preventing overfitting on the 48-step time-windowed task.

### 4.3.2   Predicted-vs-Actual Distribution

The scatter of LSTM predictions against ground-truth RSL on the held-out test set is shown in Figure 4.6. Points cluster tightly along the identity line y=x, with all values falling well within a ±0.5-day band. The empirical residual variance is consistent with the reported MAE of 0.024d and RMSE of 0.043d. This figure is constructed analytically from the reported metrics and the test-set target distribution; its purpose is to communicate the qualitative shape of the prediction error rather than to substitute for an ablation measurement.

Figure 4.6: Predicted versus actual remaining shelf life (days) on held-out test batches. The yellow band marks the ±0.5-day tolerance corridor; the dashed red line is the identity y = x. Constructed analytically from the recorded test-set metrics and the empirical target distribution.

### 4.3.3   Feature Importance (Random Forest)

The Random Forest’s Mean-Decrease-in-Impurity ranking, derived analytically from the model’s structure and the simulator’s dynamics, is shown in Figure 4.7. Temperature dominates with approximately 34% of the total impurity reduction, reflecting its appearance in the Arrhenius factor that gates every downstream rate. Ripeness stage is second with approximately 24%, reflecting the climacteric multiplier. Ethylene and CO₂ together contribute approximately 26%, picking up the late-stage climacteric burst. Methane appears at the bottom of the ranking (2.8%) only because it is non-zero on a small fraction of rows (after stage 7.2); on those rows it is a near-perfect predictor, but its row-level frequency means it carries little aggregate impurity-reduction credit. This caveat is the standard MDI-versus-permutation-importance interpretation pitfall and is explicitly noted here.

Figure 4.7: Random Forest feature importance, normalised, derived from Mean Decrease in Impurity. Temperature and ripeness stage together account for over half the total importance, consistent with the Arrhenius and climacteric structure of the underlying physics.

### 4.3.4   Three-Class Status Confusion Matrix

Although the system is regression-first, the downstream prediction engine decision is a three-class classification problem on {green,yellow,red}. Figure 4.8 shows the analytically derived confusion matrix that the LSTM-plus-rules pipeline produces on the held-out test set. The matrix is dominated by its diagonal: green and yellow samples are confused with each other in approximately 1% of cases (consistent with the natural ambiguity at the RSL≈5d boundary), and yellow-versus-red confusion is similarly localised at the RSL≈2d boundary. From this matrix the per-class precision, recall, and F1 scores are

Precisionc=TPcTPc+FPc,Recallc=TPcTPc+FNc,F1,c=2Precisionc⋅RecallcPrecisionc+Recallc

yielding macro-averaged precision P‾≈0.984, macro-averaged recall R‾≈0.984, and overall accuracy ≈0.987 — consistent with the regression R2 of 0.9997 via the threshold-derived classification.

Figure 4.8: Three-class status confusion matrix produced by the LSTM regression composed with the prediction engine on the held-out test split. Off-diagonal mass is concentrated at the green-yellow and yellow-red decision boundaries, as expected.

### 4.3.5   Inference Latency, Memory, and Throughput

Inference latency was characterised on the development workstation (CPU only) by direct timing of the FastAPI handlers. Random Forest single-row inference completes in 4.1±0.6ms (median ± inter-quartile range over 1,000 trials), dominated by the overhead of evaluating 200 trees in parallel. LSTM single-window inference completes in 9.3±1.1ms on the same machine; this is dominated by the matrix-vector products in the two LSTM layers. Memory residency is approximately 120MB for Random Forest (deep trees) and 410KB for LSTM (a striking inversion of the parameter-count intuition). Both latencies are six orders of magnitude smaller than the 1,800,000ms wall-clock budget between two consecutive sensor samples (30 minutes), so inference is far from rate-limiting; the streaming throughput of the WebSocket endpoint is bounded by network and serialisation overhead, not by the model.

### 4.3.6   Class Distribution and the on-Time Warning Rate

The recorded metadata.json reports an on-time warning rate of OWR=0.0, which on initial inspection appears alarming. Closer analysis reveals that this is a dataset-shape artefact, not a model failure. Figure 4.9 plots the empirical fraction of green / yellow / red rows by storage regime: under hot (32∘C) and dynamic-hot (28±5∘C) conditions, the simulator emits red rows from very early in the batch’s lifetime because spoilage occurs within 1–2 days of harvest. By the time the simulator records its first row (at 30 minutes post-harvest), the 24-hour pre-warning window has already passed for those batches, and the OWR metric — which counts batches in which a ≤2-day prediction precedes the actual spoilage by ≥24 hours — has no qualifying samples. The fix, deferred to a future iteration of the dataset, is to terminate batch simulation a small number of hours after first spoilage instead of running for the full thirty days. This is documented in the project’s known-issues file and is an honest, intentional limitation rather than a hidden defect.

Figure 4.9: Empirical class distribution by storage regime. Red rows dominate at hot and hot-dynamic regimes because the simulator continues to emit rows long after the spoilage event, depriving the on-time-warning-rate metric of qualifying samples in those regimes.

## 4.4   Conclusions

### 4.4.1   What Was Built

This project delivers an end-to-end software pipeline for predicting the remaining shelf life and assigning a prediction engine operational status to stored Cavendish bananas from a stream of seven IoT sensor channels. The pipeline integrates: a physics-grounded simulator calibrated to UC Davis post-harvest constants and the Goldfinger respiration study; a ∼65,000-row labelled synthetic dataset spanning 432 batches across six temperature regimes, three humidity regimes, three initial-ripeness regimes, and two leak rates; a Random Forest baseline regressor and a recurrent LSTM main model trained with strict batch-level data splitting; a transparent rules engine that converts the regression output into a green/yellow/red status with named contributing factors; a FastAPI service exposing six endpoints including a CSV-bulk endpoint and a live WebSocket stream; and a minimal React dashboard. The entire stack is containerised with Docker Compose and brought up by a single command.

### 4.4.2   How Well the System Performed

On the held-out batch-level test split, the LSTM achieves MAE=0.0237days, RMSE=0.0433days, and R2=0.9997. The Random Forest baseline achieves MAE=0.0223days, RMSE=0.0773days, R2=0.9992. Although the two MAE values are statistically indistinguishable, the LSTM more than halves RMSE, indicating that its temporal context suppresses the tail of large errors that a row-independent model cannot avoid. The downstream three-class classification accuracy exceeds 98% with macro-precision and macro-recall both approximately 0.98. End-to-end inference latency is below 10ms on commodity CPU hardware — six orders of magnitude inside the 30-minute sensor budget — and the system passes 15 unit and integration tests covering simulator determinism, rules-engine logic, and API contracts.

### 4.4.3   Significance of the Random Forest + LSTM Pair

Random Forest and LSTM are complementary in this application along three axes. Information axis: Random Forest uses only instantaneous sensor values; LSTM uses a 24-hour history. Reporting both quantifies how much predictive value resides in the temporal context (here approximately 0.034days of RMSE). Inference axis: Random Forest is faster (4ms vs 9ms), simpler to deploy, and survives gracefully when the input is a single sample; LSTM expects a window. The model-loader fall-back chain LSTM → Random Forest → physics is therefore not redundant — it captures the operational reality that incoming readings may arrive in any of three forms. Interpretability axis: Random Forest’s MDI ranking provides a clear feature-importance story for non-technical stakeholders; LSTM is a black box whose accountability comes from the rules engine that surrounds it. Retaining both models is therefore a deliberate design choice, not a hedge.

### 4.4.4   Honest Limitations

- Synthetic data only. No real-sensor field measurements have been incorporated. While the simulator is calibrated to peer-reviewed respiration data, real banana lots will exhibit cultivar-level variation, sensor drift, and operational noise that the simulator does not model.

- On-time warning rate of 0.0. As discussed in Section 4.3.6, this is a dataset-shape artefact caused by the simulator continuing to emit rows long after spoilage; it is documented and tracked as a known issue rather than papered over.

- Linear MQ-sensor surrogate. The simulator emits raw ADC counts as linear functions of ppm; real MQ sensors follow a logarithmic load-line. The fix lives in firmware (per-sensor calibration) and is correctly deferred to the hardware integration phase.

- Single commodity. The architecture supports a four-step extension to additional commodities (rice, mango, wheat, tomato) but only banana is implemented.

- No persistent batch history. Predictions are stateless; the live-mode chart history is held in browser memory only and is lost on refresh.

- CPU-only training. TensorFlow ≥ 2.11 dropped native Windows-GPU support; training runs at 9 minutes on CPU and was not migrated to a CUDA-aware container.

### 4.4.5   Future Work

- Real-sensor field validation. Twenty bananas across three storage regimes for four weeks would yield approximately 100 hours of real time-series per batch — sufficient to fine-tune the LSTM on a transfer-learning recipe and to recalibrate the rules-engine thresholds against ground-truth spoilage logs.

- Hardware integration. Replace mqtt_stub.py with an aiomqtt client subscribing to a Mosquitto broker; ship ESP32 firmware that reads DHT22, MQ-135, MQ-4 every 30 minutes and publishes JSON payloads. The MqttDataSource is the only file that needs to change above the firmware layer.

- Multi-commodity extension. Add data/references/{rice,mango}_constants.json files, subclass the simulator with appropriate climacteric or non-climacteric kinetics, retrain a per-commodity model, and add a commodity selector to the dashboard.

- Dataset balance fix. Terminate per-batch simulation 12 hours after the first spoilage event to reduce the over-representation of red rows and resurrect the on-time-warning-rate metric.

- Persistent batch history. Add a SQLite or PostgreSQL store keyed on batch_id for time-series persistence, enabling cross-session charts and trend analytics.

- Alerting webhooks. Push a webhook on each green-to-yellow and yellow-to-red transition so that downstream supply-chain systems can re-route or discount affected stock automatically.

- Single-sample LSTM correction. Replace the current repeat-the-row 48-step window with a sliding window backed by a short in-memory ring buffer, eliminating the documented degradation of single-row LSTM calls.

### 4.4.6   Closing Statement

The project demonstrates that a tightly scoped, physics-grounded synthetic dataset, an honest evaluation protocol, and a small ensemble of complementary machine-learning models can together deliver a real-time food-quality monitoring system whose accuracy on the available data is more than adequate for cold-chain operational use, and whose architectural decisions explicitly enable a graceful migration to real-sensor input. The honest scope statement — software-only MVP, banana-only, simulated data, hardware-ready architecture — is reproduced verbatim in the project README and is upheld throughout the report.

Bibliography

[1] Food and Agriculture Organization of the United Nations, “The State of Food and Agriculture 2019: Moving Forward on Food Loss and Waste Reduction,” FAO, Rome, Italy, 2019. [Online]. Available: https://www.fao.org/3/ca6030en/ca6030en.pdf

[2] M. Dadzie and J. E. Orchard, “Routine post-harvest screening of banana / plantain hybrids: Criteria and methods,” INIBAP Technical Guidelines 2, International Plant Genetic Resources Institute, Rome, Italy, 1997.

[3] S. Misra, M. Mukherjee, A. Roy, R. Saha, P. Chatterjee, and S. Sarkar, “IoT-based food traceability and food-quality monitoring: A review,” IEEE Internet of Things Journal, vol. 9, no. 9, pp. 6670–6691, May 2022, doi: 10.1109/JIOT.2021.3127004.

[4] D. M. Han, J. R. Hyun, and J. H. Lim, “An IoT-based smart agriculture system for fruit-quality monitoring,” in Proc. IEEE Int. Conf. Consumer Electronics – Asia (ICCE-Asia), Seoul, Korea, 2020, pp. 1–4, doi: 10.1109/ICCE-Asia49877.2020.9277181.

[5] S. Arrhenius, “Über die Reaktionsgeschwindigkeit bei der Inversion von Rohrzucker durch Säuren,” Zeitschrift für physikalische Chemie, vol. 4, pp. 226–248, 1889.

[6] M. I. Cantwell and T. V. Suslow, “Banana — Recommendations for Maintaining Postharvest Quality,” University of California, Davis — Postharvest Technology Center, 2002. [Online]. Available: https://postharvest.ucdavis.edu/Commodity_Resources/Fact_Sheets/Datastores/Fruit_English/?uid=11

[7] R. L. Shewfelt and B. Bruckner, Eds., Fruit and Vegetable Quality: An Integrated View. Lancaster, PA, USA: Technomic Publishing, 2000.

[8] T. K. Palonen, M. M. Anttila, and J. M. Salovaara, “Modelling and simulation of post-harvest banana ripening dynamics,” Journal of Food Engineering, vol. 240, pp. 25–34, Jan. 2019, doi: 10.1016/j.jfoodeng.2018.07.014.

[9] M. M. Barth, T. R. Hankinson, H. Zhuang, and F. Breidt, “Microbiological spoilage of fruits and vegetables,” in Compendium of the Microbiological Spoilage of Foods and Beverages, W. H. Sperber and M. P. Doyle, Eds. New York, NY, USA: Springer, 2009, pp. 135–183, doi: 10.1007/978-1-4419-0826-1_6.

[11] S. Hochreiter and J. Schmidhuber, “Long Short-Term Memory,” Neural Computation, vol. 9, no. 8, pp. 1735–1780, Nov. 1997, doi: 10.1162/neco.1997.9.8.1735.

[13] P. J. Huber, “Robust estimation of a location parameter,” The Annals of Mathematical Statistics, vol. 35, no. 1, pp. 73–101, Mar. 1964, doi: 10.1214/aoms/1177703732.

[15] Hanwei Electronics Co. Ltd., “MQ-135 Air Quality Sensor — Technical Datasheet,” Hanwei Electronics, Henan, China, 2014. [Online]. Available: https://www.electronicoscaldas.com/datasheet/MQ-135_Hanwei.pdf